FETAKGOMO WATER SECTOR PLAN 
TABLE OF CONTENTS

Contents:            …………………………………………..2-3

Introduction:    …………………………………………..4-4

Status Analysis: .…………………………………………5-5

Risk Analysis: ……………………………………………7-7

Quality and Challenges: ..………………………………8-8

Recommendations: . …………………………………...9-10

Analysis per Ward: ………………………………….11-71

INTRODUCTION

Background

Fetakgomo has been recognized as an area where water availability is very limited. Previously DWAF was responsible for planning, implementation, operation and maintenance of water schemes.

Most of community water supplied developed from groundwater source with boreholes, operated by diesel, petrol or electric motors

Challenges:

- Breakdowns

- Lack of accountability in operation and maintenance

- No involvement of the communities in the O&M of their schemes

- Lack of information in terms of implemented projects- i.e. as –built drawings

- Lack of monitoring data

GOALS AND OBJECTIVES

The Goal

- The overall goal to develop , ensure and  manage  availability  of portable  water  within all areas ,also for development and  management of the available water sources in sustainable manner .

- To provide adequate sanitation facilities

- To provide all communities with sustainable water supplies, sufficient for their needs, equitably distributed within the available resources.

The objective

- The integration of water sector planning with overall development needs as outlined in the integrated Development plans

- The balancing of various environmental requirements with the requirements of water for economic development ,with  particular attention being focused on effects of water quality resulting from mining operations, irrigation, on site  sanitation and underground recharge.

- To meet various requirements interims of water and sanitation provision .

- To meet national target of 2008

STATUS ANALYSIS & LEVEL OF SERVICE

Water

Level 0 service: is a temporary kind of service.

Level 0: services provided strictly to critical areas based on the followings:

- Area without any existing source

- Break down schemes

- Cut of water

Level 1: immediate mitigation to address category/level 0

Level 2: Reticulation – below RDP –minor retics to address Level 1

Level 3: Reticulation -Up to RDP- full reticulation to address Level 0-2, to meet all recommended RDP standard, e.g. recommended traveling distance of 200m.

Level 4: Yard Connections –high level of services as a mitigation to all above mentioned levels.

LEVEL OF SERVICE:

| LEVEL | FUNCTIONALITY | COMPLIANCE WITH RDP STANDARDS | O&M | COST RECOVERY |
| --- | --- | --- | --- | --- |
| Level 0: Emergency Water Provision | For Critical Areas | No infrastructure |  | None |
| Level 1: Drought Relieve Schemes | To alleviate level 0 | Does not meet all RDP requirements | Response time less than 2 days | None |
| Level 2:Reticulation Below RDP | Functional 60 % of time | May meet 4 out of 5 RDP requirements | Response time less than 2 days | None |
| Level 3:Reticulation -above RDP | Functional 90 % of time | Meet all RDP requirements | Response time less than 2 days |  |
| Level 4: Yard connection | Functional 98% 0f time | Meet all RDP requirements | Response time less than 2 days | Meter reading ;control of private connections. |

RISK ANALYSIS

- SOURCES & Infrastructure

The municipality has only two water source, which are as follows:

- Surface Water

- Ground Water

Regardless of these available sources, the municipality is encountering a very critical situation in terms of availability of water.

GROUND WATER:

It plays major role  as a water resource for most of the communities within Fetakgomo Municipality

YIELD & CHALLENGE

It has been proven that 88% of boreholes have drilled to alleviate drought in Fetakgomo, but in vain, as their yields have tested drastically low or totally dry.

Yield varies considerably from less than 0.5 l/sec to 5l/sec.

EFFECTS/INFLUENCES:

- There are very real effects of mining activities, which impact, negatively on the water resources and on the future infrastructure plan.

- Effects of the mineralization of ground water resources due to discharge of mines

Due to the above, 90% of municipal water schemes end up in drought (95%most of municipal schemes depend on under ground water)

QUALITY & CHALLENGE

Water quality tested salty, pH is between ………. and is not good for human consumption or for domestic usage.

According to Sekhukhune Infrastructure Plan ,ground water in areas next to mines is generally of Class 2 or Class 3 due to high level of Nitrate, high Magnesium hardness high level of fluoride.

SURFACE WATER

Olifantspoort is the only catchment identified and regional water supply scheme supplied from surface water source . Bulk line has been constructed since 2000 to reach some villages, i.e. Nkwana,Nchabeleng, Lerajane, Apel ,Pelangwe,India,Malogeng,Modimolle,Mphaaneng.

Olifantspoort bulk line continues up to the following villages:

- Mohlaletse; Sekhukhune College; Matlala; Thabanaseshu; Thobehlale

Challenge:

- Lack of Infrastructure

Demand:

To obtain meaningful data on the expected water demand  within municipality

95% of villages still to be covered by the scheme.

RECOMMENDATIONS:

GROUND WATER:

Yield & Quality

- Hydrological assessment should take place before drilling.

- Yield test

- Quality test

SURFACE WATER

- Provision of adequate infrastructure to all villages to minimize  in vain drilling of boreholes.

- INFRASTRUCTURE:

WATER

RECOMMENDATIONS:

  - EXISTING WATER SCHEMES:

- Augment all existing schemes up to the recommended standard.

- O&M –turn around time response to be improved up to  at least within 48 hours.

- PROJECTS UNDER CONSTRUCTION

- Completion of all  stalled projects

- Future O&M  plan

- Quality assurance

- Project Monitoring

3. SANITATION

Status:

Simple Latrine

- Assessment:

According to assessments made by Municipal Officials during January 2007, most of the existing simple latrines are in a very malicious condition.

- Challenges:

- Most of structures are mud and Zink latrines.

- Recommendation

- All simple latrines are assessed below RDP standard and recommended for an upgrade up to RDP Standard.

- All facilities should comply with disability standard.

- Feasibility Studies to be executed before commencement of any physical work.

ANALYSIS PER WARDS

DOMESTIC/ HOUSHOLDS WATER: WARD 1

| VILLAGES SUB-VILLAGES |  | NUMBER OF HOUSEHOLDS |  |  | EXISTING INFRASTRUCTURE |  |  |  |  |  |  |  |  |  |  | BOTTLENECKS |  | STRATEGY |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | STORAGE & STAND PIPES |  | NUMBER OF BOREHOLES/ SOURCE |  |  | SOURCE OF ENERG | ABOVE RDP |  | BELOW RDP |  |  |  |  |  |  |  |
| Phahlamanoge Masehleng |  | 362 |  |  | Drought relief structure (two tanks and 1 x stand pipe ,2 taps) |  |  |  |  | Diesel |  |  | Yes |  |  | Two water tanks, one dysfunctional .Tanks stand not fixed/buckle Communities do not receive water due to limited water pressure Project below RDP standard Source not reliable |  | Refurbish existing water schemes to re-establish effective and efficient water supply (existing water and sanitation infrastructure): Installation of booster pump Provision of new stand Augment the source by drilling of a new borehole and connect to the existing inlet O&M- to sustain scheme Electrification of borehole |  |  |
| Kanana |  |  |  |  | No infrastructure |  | No borehole |  |  |  |  |  |  |  |  | No source of water |  | Provide water reticulation –Extension(pressure to be confirmed) New reticulation and drilling and equipping of borehole |  |  |
| Sefateng |  |  |  |  | Drought relief structure (one tank and 2xstand pipe) |  | Borehole |  |  | Diesel |  | Yes |  |  |  | Water yield/slope |  | Slope: Installation of booster pump/ Drilling of a new borehole( to avoid cost) Yield: Drilling of a new borehole |  |  |
| Mmabatho |  |  |  |  | Village is below RDP level Main line Water not sufficient |  |  |  |  |  |  |  |  |  |  | Main line does not cover the whole village Source not reliable |  | Upgrade the existing line Reticulate |  |  |
| Setopong |  |  |  |  | Main line | Borehole |  |  |  | Diesel |  | Yes |  |  |  | Main line does not cover the whole village Source not reliable |  | Upgrade the existing line up to RDP Standard Reticulation |  |  |
| Kgwaripane |  |  |  |  | Main line | Borehole |  |  |  | Diesel |  | Yes |  |  |  | Main line does not cover the whole village Source not reliable |  | Extend the main line to out reach the whole village Reticulate |  |  |
| Moshate |  |  |  |  | Village is below RDP level. Main line. | borehole |  |  |  | Diesel |  | Yes |  |  |  | Main line does not cover the whole village Source not reliable |  | Extend the main line to out reach the whole village Reticulate |  |  |
| Difoeng |  |  |  |  | Main line |  |  |  |  |  |  |  |  |  |  | Main line not functional Below RDP standard |  | Upgrade the existing pipe line Reticulate the village up to RDP standard |  |  |
| Summary | Total Number of stand pipes :40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mashilabela:Shenyaneng |  |  | 303 | Main line from Radimmela. 4 x standpipes |  |  | 1 x borehole |  | Diesel |  |  | Yes |  |  | Source not reliable |  |  |  |  | Augment the capacity of the borehole by: Drilling a new borehole Erection of new main line Install booster pump |
| Lekubeng/Manoge |  |  |  | Main line from Manoge 5 x stand pipes |  |  |  |  | Diesel |  |  | Yes |  |  | Existing line not functional and does not cover all households |  |  |  |  | Extend the existing line Additional Storage /Tanks Provide reticulation –up to RDP standard Ensure adequate supply from Manoge |
| Sekhutlong |  |  |  | No infrastructure invested, only one hand pump available |  |  |  |  |  |  |  |  |  |  | Pump not functional |  |  |  |  | Provide a RDP standard reticulation |
| Lehlabile |  |  |  | Borehole 3 x Stand pipe |  |  |  |  | Electricity |  |  | Yes |  |  | Below RDP standard. Quality of water not satisfactory. |  |  |  |  | Upgrade the existing scheme up to RDP standard Improve quality of water up to an acceptable pH (test to be done to check a suitable technique ) |
| Ga-Oria |  |  |  | 5 x stand pipe 2 hand pump |  |  |  | Borehole | Electricity |  |  | Yes |  |  |  |  |  |  | Upgrade the existing scheme up to RDP standard Improve quality of water up to an acceptable pH (test to be done to check a suitable technique ) |  |
| Mohlaletse: Maebe |  |  | 1282 | No infrastructure |  |  |  |  | None |  |  |  |  | No provision of service |  |  |  |  | New water reticulation Source : Olifantspoort South bulk line |  |
| Lokasi |  |  |  | No infrastructure |  |  |  |  | None |  |  |  |  | No provision of service |  |  |  |  | New water reticulation Source : Olifantspoort South bulk line |  |
| Shushumela :Next to Thobejane |  |  |  | Old Pipe line constructed by former Lebowa |  |  |  |  | Diesel |  |  |  |  |  |  |  |  |  |  |  |
| Fetakgomo |  |  |  | Ditto |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GaMakola/ GaMatji |  |  |  | Borehole Main line with stand pipes |  |  |  |  | Diesel |  |  |  |  | Below RDP standard. Water system dilapidated . |  |  |  |  | Provide water reticulation- up to RDP standard Rehabilitation of the existing scheme Source : Olifantspoort South bulk line |  |
| Fetakgomo |  |  |  | Borehole. Main line with stand pipes |  |  |  |  | Diesel |  |  |  |  | Below RDP standard Water system dilapidated |  |  |  |  | Provide water reticulation-up to RDP standard. Rehabilitation of the existing scheme Source : Olifantspoort South bulk line |  |

DOMESTIC /HOUSEHOLDS SANITATION INFORMATION

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLDS | SANITATION EXISTING INFRASTRUCTURE |  | NUMBER OF HOUSEHOLDS WITHOUT LATRINES | CHALLENGES | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | VIP/RDP | SIMPLE LATRINE/BELOW RDP |  |  |  |
| Phahlamanoge: Masehleng | 362 | 50 | 85 | 227 |  | Feasibility studies –Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| Kanana |  |  |  |  |  |  |
| Sefateng |  |  |  |  |  |  |
| Mmabatho |  |  |  |  |  |  |
| Setopong |  |  |  |  |  |  |
| Kgwaripane |  |  |  |  |  |  |
| Moshate |  |  |  |  |  |  |
| Difoeng |  |  |  |  |  |  |
| Mashilabela:Shenyaneng | 303 | 79 | 94 | 130 |  |  |
| Lekubeng/Manoge |  |  |  |  |  |  |
| Sekhutlong |  |  |  |  |  |  |
| Lehlabile |  |  |  |  |  |  |
| Ga-Oria |  |  |  |  |  |  |
| Mohaletse : Maebe Shushumela | 1282 | 93 | 1220 | 1189 |  |  |
| Lokasi Matji Makola |  |  |  |  |  |  |
| Fetakgomo |  |  |  |  |  |  |

SCHOOLS   WATER AND SANITATION PROVISION INFORMATION

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY |  | ABOVE RDP | BELOW RDP | RDP/ABOVE RDP | Simple Latrine/BELOW RDP |  |
| Mashilabela Primary | Tanks Stand pipe | Borehole |  | No electricity |  |  |  |  | Additional sanitation facilities required to meet RDP standard Electrification of borehole |
| Mokhine High School | Tanks Stand pipes | Borehole |  | Diesel |  |  |  |  | Additional sanitation facilities required to meet RDP standard |
| Maebe Primary | Bulk |  |  | Electricity | Fair |  |  |  | Additional sanitation facilities required to meet RDP standard |
| Masehleng Primary | ? | Borehole |  | Electricity | Fair |  |  |  |  |
| Mohlaletse Sec | Stand pipe | bulk |  | Electricity |  |  |  |  | Provision of adequate sanitation facilities |
| Moletse Primary | No infrastructure |  |  |  |  |  |  |  | Provision of water and sanitation facilities |
| Mankopodi | No infrastructure |  |  |  |  |  |  |  | Provision of water and sanitation facilities |

HEALTH FACILITIES  EXISTING INFRASTRUCTURE : WATER AND SANITATION

| CLINIC NAME | WATER EXISTING INFRASTRUCTURE |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  |  |  |  |  | REMARK |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | ENERGY | ABOVE RDP | BELOW RDP | SIMPLE LATRINE/BELOW RDP | VIP/RDP | SEPTIC TANK /WATER BORN | ABOVE RDP | BELOW RDP |  |  |
| Phahlamanoge Clinic | Under construction | Not energized | Yes |  | 2 | 15 |  | Yes |  | Clinic still under construction |  |

DOMESTIC/ HOUSHOLDS WATER:WARD 2

| VILLAGES SUBVILLAGES |  | NUMBER OF HOUSEHOLDS | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | BOTTLENECKS | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | STORAGE & STAND PIPES | NUMBER OF BOREHOLES/ SOURCE | SOURCE OF ENERG | ABOVE RDP | BELOW RDP |  |  |
| Mphanama Sepakapakeng |  | 2505 | Main line Reticulation |  |  | YES |  | Capital project failed Main line not functional and no adequate source | Erection of bulk line from Olifantspoort south (to be cost effective) Connect the existing infrastructure to Nkadimeng dam |
| Ga- Matebane Magabaneng |  |  |  |  |  |  |  |  |  |
|  |  |  | Main line Reticulation | Borehole | Electricity | Yes |  | Capital project failed | Connect to Olifantspoort south Connect the existing line to Nkadimeng dam |
| Magagamatala Seleteng Matamong Malaeneng |  |  |  |  |  |  |  |  |  |
|  |  |  | Main line not functional Reticulation | No source | Diesel | Yes |  | Capital project failed | No source Main line not functional Reticulation connect the existing line to Nkadimeng dam |
|  |  |  | Main line not functional Reticulation | No source | Diesel |  |  | Main line does not cover the whole village Source not reliable | Connect to Olifantspoort south Extension of line to cover up the area which are not covered. Connect the existing line to Nkadimeng dam |
| SUMMARY | TOTAL NUMBER OF STANDPIPES: 61; Functional standpipes 23.PUMP HOUSE: 5;RESERVOIR:3;BOREHOLE:5 |  |  |  |  |  |  |  |  |

DOMESTIC /HOUSEHOLDS SANITATION INFORMATION

| VILLAGES SUBVILLAGES | NUMBER OF HOUSEHOLDS | SANITATION EXISTING INFRASTRUCTURE |  | SANITATION EXISTING INFRASTRUCTURE BACKLOG | STRATEGY |
| --- | --- | --- | --- | --- | --- |
|  |  | VIP/RDP | SIMPLE LATRINE/BELOW RDP |  |  |
| Mphanama Sepakapakeng | 2505 | 194 | 1166 | 912 | Feasibility studies -Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| GaMatebane |  |  |  |  |  |
| Magagamatala Matebene |  |  |  |  |  |
| Malaeneng Seleteng Matamong |  |  |  |  |  |

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  | STRATEGY |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | VIP/ABOVE RDP | SIMPLE LATRINE/BELOW RDP |  |  |
| Makelepeng Secondary | 1 x tank | Mobile Water tanker |  |  |  |  |  |  | Provide water infrastructure as per RDP standard |
| Dithotweneng Sec | 2 x tanks, taps | Dysfunctional | Electricity |  |  |  |  |  | Additional sanitation facilities required to meet RDP standard |
| Modipadi Primary | 2x tanks, taps | Borehole | Electricity |  |  |  |  |  | Additional sanitation facilities required to meet RDP standard |
| Mphanama H. Primary | 2 x tanks, taps | Community Borehole | Electricity |  |  |  |  |  | Additional sanitation facilities required to meet RDP standard |
| Maboe H -Primary | No existing infrastructure |  |  |  |  |  |  |  | Provide water infrastructure as per RDP standard |
|  |  |  |  |  |  |  |  |  | Provide water infrastructure as per RDP standard |

SCHOOLS   WATER PROVISION INFORMATION

HEALTH FACILITIES EXISTING INFRASTRUCTURE: WATER AND SANITATION

| CLINIC NAME | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | SIMPLE LATRINE/ BELOW RDP | VIP/ ABOVE RDP | SEPTIC TANK/ WATER BORN |  |
| Mphanama Clinic | Minor retic | Borehole | Electricity | Yes |  | 4 |  | 1 | Project still under construction |

DOMESTIC/ HOUSHOLDS WATER:WARD 3

| VILLAGES SUBVILLAGES |  | NUMBER OF HOUSEHOLDS | WATER EXISTING INFRASTRUCTURE |  |  |  |  |  |  |  | BOTTLENECKS |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | STORAGE & STAND PIPES | NUMBER OF BOREHOLES/ SOURCE |  | SOURCE OF ENERG |  | ABOVE RDP | BELOW RDP |  |  |  |  |
| Seroka: Matlou Seroka Village |  | 716 | Main line Two Tanks 4 x Stand pipes 80% of Unauthorized Yard connections | 1x borehole |  | 1x Patrol 2 x Diesel |  |  | Yes |  | no adequate source |  | Upgrade the existing up to RDP standard Source: Olifantspoort bulk line O&M –to sustain the scheme |
| Malaeneng |  |  | Main Line | Borehole |  | Electricity |  |  | Yes |  |  |  | Upgrade the existing up to RDP standard Source: Olifantspoort bulk line O&M –to sustain the scheme |
| Moshate |  |  | Main line not functional | No source |  |  |  |  | Yes |  |  |  | Upgrade existing up to RDP standard Source: Olifantspoort bulk line O&M –to sustain the scheme |
| Summary :Total Number of standpipes: 327,Borehole :3;Reservoir : 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Malekaskraal | Tswereng | 124 | 25 x stand pipe | 3x Borehole |  | Electricity |  |  | Unauthorized yard connection |  |  | Upgrade the existing up to RDP standard Source: Olifantspoort bulk line O&M –to sustain the scheme |  |
|  | Makopa | 71 | 53 x standpipes | Borehole 1x pump house |  | Electricity |  |  |  | Illegal connection Below RDP standard |  |  |  |
|  | Sekhotlong | 83 | 4x stand pipes 4x tanks Hand pump |  |  | Diesel |  |  | Yes |  |  | - Augment the existing infrastructure up to RDP standard |  |
| MOHLALATSE: |  |  | Water reticulation |  |  | Electricity |  |  |  | Completion of pump station |  | - Upgrade the existing up to RDP standard Source: Olifantspoort bulk line O&M –to sustain the scheme |  |
| Ga-Phasha |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mapulaneng |  |  | Water reticulation |  |  | Electricity |  |  |  | Completion of pump station |  |  |  |
| Debeila |  |  | Borehole |  | Bulk |  | Yes |  |  |  |  |  |  |
| Mohlaletse: Tjate Moshate Molalaneng |  |  | Bulk Line |  | Bulk |  |  |  | Yes | Bulk does not cover the area |  | - Upgrade the existing up to RDP standard Source: Olifantspoort bulk line O&M –to sustain the scheme |  |

HOUSEHOLDS SANITATION INFORMATION

| VILLAGES SUBVILLAGES | NUMBER OF HOUSEHOLDS | SANITATION EXISTING INFRASTRUCTURE |  |  | SANITATION INFRASTRUCTURE BACKLOG | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | VIP/RDP | SIMPLE LATRINE/BELOW RDP |  |  |  |
| Seroka Matlou Malaeneng Moshate | 716 | 102 |  | 204 | 410 | Feasibility study- Geotech Provide each family with ventilated prefabricated latrine |
| Malekaskraal Tswereng | 124 | 51 |  | 72 | 5 |  |
| Makopa | 71 | 61 |  | 19 |  |  |
| MOHLALETSE: Phasha Mapulaneng Debeila | 26 | 1 |  | 13 | 12 |  |
| Tjate Moshate Molalaneng |  |  |  |  |  |  |

SCHOOLS   WATER AND SANITATION PROVISION INFORMATION

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION STATUS |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | ABOVE RDP |  | BELOW RDP |  |
| Peu Secondary | Stand pipes | Borehole | Electricity |  | Yes | Yes |  |  | Additional sanitation facilities required to meet RDP standard |
| Matleu Primary | Standpipes | Borehole | Electricity |  | Yes | Yes |  |  | Additional sanitation facilities required to meet RDP standard |
| Seroka Primary | Stand pipes | borehole | Electricity |  | Yes | Yes |  |  | Additional sanitation facilities required to meet RDP standard Provision of water infrastructure |
| Thete Secondary |  |  |  |  |  |  |  |  |  |
| Moshiane primary |  |  |  |  |  |  |  |  |  |
| Mampuru Thulare Primary |  |  |  |  |  |  |  |  |  |
| Leganabatho Primary |  |  |  |  |  |  |  |  |  |
| Dinakanyane Secondary | Stand pipe | Mainline | Electricity |  | Yes | Yes |  |  | Additional sanitation facilities required to meet RDP standard Provision of water infrastructure |

| CLINIC NAME | WATER EXISTING INFRASTRUCTURE |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | SIMPLE LATRINE/ BELOW RDP | VIP/ ABOVE RDP | SEPTIC TANK/WATERBORN |  |
| Seroka Clinic | 2 x Stand pipes ,2x jojo tanks, Minor retic | Borehole | Electricity | Fair |  |  | 4 | 4 | Upgrading of the existing water infrastructure. |
| Mohlaltse Clinic | 1 x tank , 2 x stand pipes | Borehole | Electricity |  | Yes | 2 |  |  |  |

HEALTH FACILITY WATER AND SANITATION PROVISION INFORMATION

DOMESTIC  /HOUSHOLDS WATER :WARD 4

| VILLAGES SUBVILLAGES | NUMBER OF HOUSEHOLDS | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | BOTTLENECKS | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | STORAGE & STAND PIPES | NUMBER OF BOREHOLES/ SOURCE | SOURCE OF ENERG | ABOVE RDP | BELOW RDP |  |  |
| Ga-Radingwana- | 279 | 10 stand pipes Reticulation 2x Pump house 3 x reservoir | 2 borehole | Electrified Diesel |  |  |  | Augment existing boreholes |
| GaMmela | 180 | Reticulation at RDP standard, 1 x Borehole and 3 x tanks 19 x stand pipes |  | Electricity and Diesel |  |  |  | Scheme up graded up to RDP Standard. |
| Phageng | 150 | Mainline existing 9 x stand pipe 2 x pump house |  | Electricity Diesel |  |  | Stand pipes not enough | Provision of addition required stand pipes |
| Ga-Mohlala | 253 | Mainline existing 13 x Stand pipe |  | Electricity |  |  | Below RDP Standard | Upgrade the existing scheme-up to RDP standard |
| Mohlaletse:Rite | 422 | Borehole and four tanks Old reticulation 3x stand pipes 1x borehole for livestock’s |  | Diesel |  |  | Borehole dysfunctional Below RDP standard | Upgrade the existing scheme-up to RDP standard Source :Olifantspoort bulk line |
| Lekgwarapaneng/Malaeneng | 254 | Old reticulation |  | Diesel |  |  | Below RDP standard Main line partially functioning |  |

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLDS | SANITATION EXISTING INFRUSTRUCTURE & STATUS |  | INFRASTRUCTURE BACKLOG | STRATEGY |
| --- | --- | --- | --- | --- | --- |
|  |  | VIP/RDP | SIMPLE LATRINE/BELOW RDP |  |  |
| Ga- Radingwana: Section E | 151 |  |  |  | Feasibility studies -Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| Section F | 128 |  | 128 |  |  |
| Section C | 95 |  | 95 |  |  |
| GaMmela: Section D | 180 |  | 180 |  |  |
| Phageng: Section B | 150 |  | 150 |  |  |
| Ga-Mohlala: Marokolong | 146 |  | 95 | 51 |  |
| Motshana |  |  |  |  |  |
| Mohlaletse:Rite | 422 | 35 | 290 | 97 |  |
| Lekgwarapaneng/Malaeneng | 254 | 55 | 290 | 91 |  |

DOMESTIC/HOUSEHOLDS SANITATION

SCHOOLS   WATER AND SANITATION PROVISION INFORMATION

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP |  | VIP/ RDP |  | SIMPLE LATRINE/BELOW RDP |  |
| Mohwaduba High | 1 x stand pipe | Main line | Electricity ,Diesel |  | Yes |  |  | Yes |  | Additional sanitation facilities required to meet RDP standard Provision of water infrastructure |
| Radimmela Primary | 1 x stand pipe | Main line | Electricity , Diesel |  | Yes |  |  | Yes |  | Additional sanitation facilities required to meet RDP standard Provision of water infrastructure |
| Mmotong Primary | Standpipes | Main line | Electricity |  | Yes |  |  |  |  | Provision of water infrastructure |
| Tlou-Nare Secondary | Standpipes | Main line | Electricity |  | Yes | Yes |  |  |  | Additional sanitation facilities required to meet RDP standard Provision of water infrastructure |
| Telelo Secondary | Standpipes | Main line | Electricity |  | Yes |  |  | Yes |  | Additional sanitation facilities required to meet RDP standard Provision of water infrastructure |
| Maphuthe Primary | Tank, 2 x standpipes | borehole | Electricity . Petrol |  | Yes |  |  | 4 |  | Additional sanitation facilities required to meet RDP standard. Provision of water infrastructure. |

HEALTH FACILITIES  EXISTING INFRASTRUCTURE : WATER AND SANITATION

| CLINIC NAME | WATER EXISTING INFRASTRUCTURE &STATUS |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  |  | REMARK & CHALLENGES | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | SIMPLE LATRINE BELOW RDP / | VIP /RDP | SEPTIC TANK/WATER BORN |  |  |
| Ikageng Clinic | Stand pipes | Stand pipe | Electricity |  | Yes |  | 3 |  | Unreliable source | Augment the existing water infrastructure |

DOMESTIC/ HOUSHOLDS WATER: WARD 5

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLDS | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | BOTTLENECKS | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | STORAGE & STAND PIPES | NUMBER OF BOREHOLES/ SOURCE | SOURCE OF ENERG | ABOVE RDP | BELOW RDP |  |  |
| GaNchabeleng: | 287 | Bulk Line. 9x stand pipes. 8x hand pumps. | Bulk | Electricity |  | Yes |  | Reticulation –up to RDP standard Yard connection |
| 1. Mokhulwane |  |  |  |  |  |  |  |  |
| 2. Ditlokwe | 100 | No source |  |  |  |  | No infrastructure | Reticulation –up to RDP standard Source : Olifantspoort south bulk |
| 3. Ga-Tjebane |  | Reticulation ,stand pipes | Bulk | Electricity |  | Yes |  | Augment the existing scheme |
| MOHLALETSE: | 662 | Reticulation 10 Stand pipes 2 x Reservoirs | 4x Borehole | Diesel |  | Yes |  | Augment Reticulation –up to RDP standard Source : Olifantspoort bulk Completion of Olifantspoort pump station |
| 1. Maroteng |  |  |  |  |  |  |  |  |
| 2. GaTladi Photo |  | Reticulation | Bulk | None |  | Yes | Not functional awaiting for completion of pump station |  |
| 3. Sekateng | 186 | 8 x Standpipes 1x pump station | Bulk | Electricity None |  | Yes |  |  |
| 4. Mesopotamia |  | No infrastructure |  | None |  |  |  | Provision of water facility up to RDP standard |
| 5 .Lerajane | 430 | Main line(bulk line) 13 x Standpipes 1x pump house 2 x reservoirs | Bulk | Electricity |  | Yes |  | Extended /Augment Reticulation –up to RDP standard Source : Olifantspoort bulk |

DOMESTIC /HOUSEHOLDS SANITATION INFORMATION

| VILLAGES SUBVILLAGES | NUMBER OF HOUSEHOLDS | SANITATION EXISTING INFRASTRUCTURE & STATUS |  | SANITATION INFRASTRUCTURE BACKLOG | STRATEGY |
| --- | --- | --- | --- | --- | --- |
|  |  | VIP/RDP | SIMPLE LATRINE/BELOW RDP |  |  |
| GANCHABELENG 1. Mokhulwane | 287 | 60 1xincomplete | 175 | 52 | Feasibility studies -Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| 2. Mashaku | 96 | 19 | 61 | 71 |  |
| 3. Ditlokwe | 100 | 20 | 25 | 55 |  |
| MOHLALETSE | 662 | 95 | 307 | 260 |  |
| 1. Maroteng/Photo |  |  |  |  |  |
| 2. Sekateng | 186 | 7 | 133 | 46 |  |
| 3. Rite | 255 | 13 | 155 | 87 |  |
| 4. GaTladi Photo |  |  |  |  |  |
| 5. Phasha | 423 | 187 | 89 | 147 |  |
| 6. Lerajane | 430 | 22 | 305 | 103 |  |

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | STORAGE & STAND PIPES | NUMBER OF BOREHOLES/ SOURCE | SOURCE OF ENERG | ABOVE RDP | BELOW RDP | VIP/ABOVE RDP |  | SIMPLE LATRINE/BELOW RDP |  |
| Mokhulwane Secondary | Stand pipes | Borehole Bulk | Electricity |  |  |  |  |  |  |
| Hans Komane Secondary | 1 x stand pipe | Borehole Bulk | Electricity |  |  |  |  |  |  |
| Tseke Primary | Stand pipes | borehole | Electricity |  |  |  | Fair |  | Provision of adequate sanitation infrastructure |
| Phutakwe Secondary | Stand pipes | Borehole | Patrol Electricity |  |  |  |  |  | Provision of additional adequate sanitation infrastructure |
| Moroamoche Primary | Stand pipe, tanks | borehole | Petrol |  |  |  |  |  | Provision of adequate sanitation infrastructure |
| Lerajane Primary |  | Hand pump |  |  |  |  |  |  | Provision of adequate sanitation infrastructure Provision of water infrastructure |

SCHOOLS   WATER AND SANITATION PROVISION INFORMATION

DOMESTIC / HOUSEHOLD: WATER 6

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLD | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  |  | BOTTLENECKS | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | STORAGE & STAND PIPES |  | NUMBER OF BOREHOLES/ SOURCE | SOURCE OF ENERG | ABOVE RDP | BELOW RDP |  |  |
| GaNchabeleng: 1. Mashung | 735 | Bulk Line |  | Bulk | Electricity | Yes |  | RDP standard | Yard Connection |
| 2. Debeila |  | Bulk Line, 2x Borehole |  | Bulk |  | Yes |  | RDP standard | Yard Connection |
| 3. Moshate:Moshate river site | 239 | Bulk Line 4x standpipe |  | Bulk |  | Yes |  |  | Yard Connection Extend the Reticulation |
| 4. Ratshitsana | 134 |  |  |  |  |  |  |  |  |
| 6. Manganeng |  |  |  |  |  |  |  |  |  |
| 7. Majakaneng |  |  |  |  |  |  |  |  |  |
| 8. Masioneng |  |  |  |  |  |  |  |  |  |
| 9 .Ga tjebane | 30 | Bulk Line |  | Bulk |  |  | Yes |  | Reticulation –RDP standard Yard connection |
| Mabopo |  | No source No existing infrastructure |  |  |  |  |  |  | Reticulation –RDP standard Long term Solution : Yard connection Source : Olifantspoort bulk line |
| Makgaleng | 61 |  |  | Bulk | Electricity |  | Yes | Slope | Reticulation –RDP standard Long term Solution : Yard connection Source : Olifantspoort bulk line |
| Manganeng | 14 | Line from the bulk line |  | Bulk | Electricity |  | Yes | Slope | Reticulation –RDP standard Long term Solution : Yard connection Source : Olifantspoort bulk line |
| Ga matlala:Legopane | 81 | Reticulation |  | 1x Borehole | Diesel |  | Yes | Borehole not reliable | Short term Solution : Electrification of borehole Augment the existing scheme by providing additional stand pipes Long Term Solution Provision of bulk supply |
| Mashabela | 40 | 1x Borehole Pump Reticulation |  | 3 x standpipes 3 x tanks | Diesel |  | Yes |  | Short term Solution Electrification of borehole Long Term Solution : Provision of bulk supply |
| Thabana a Seshu |  | 1xBorehole reticulation |  |  | Diesel |  | Yes | Scheme dysfunctional | Short term Solution : Electrification of borehole Augmentation of the existing scheme Long Term Solution Provision of bulk supply |
| Thobehlale | 21 | Borehole 2x standpipes | 2x pump house |  | Diesel |  | Yes |  | Short term solution Electrification of borehole Long Term Solution : Provision of bulk supply |

DOMESTIC /HOUSEHOLDS SANITATION INFORMATION

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLDS | SANITATION EXISTING INFRASTRUCTURE & STATUS |  | SANITATION INFRASTRUCTURE BACGLOG | STRATEGY |
| --- | --- | --- | --- | --- | --- |
|  |  | VIP/RDP | SIMPLE LATRINE/BELOW RDP |  |  |
| GANCHABELENG: 1. Mashung | 735 | 350 | 548 |  | Feasibility studies -Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| 2. Debeila |  |  |  |  |  |
| 3. Moshate/ Moshate river site | 239 | 110 | 100 | 29 |  |
| 4. Ratshitsana | 134 | 18 | 93 | 23 |  |
| 5. Manganeng |  |  |  |  |  |
| 6. Majakaneng |  |  |  |  |  |
| 7. Masioneng |  |  |  |  |  |
| 8. Makgaleng | 61 | 10 | 43 + 2 unfinished | 6 |  |
| 9. Ga tjebane | 30 | 6 | 22 | 2 |  |
| 10. Mabopo |  |  |  |  |  |
| 11. Manganeng | 14 | 3 | 11 |  |  |
| GA- MATLALA: 1. Legopane | 81 | 15 | 57 |  |  |
| 2. Mashabela | 40 | 3 | 25 |  |  |
| 3. Thabana a Seshu |  | 16 | 8 |  |  |
| 4. Thobehlale | 21 | 3 | 7 |  |  |

SCHOOLS   WATER AND SANITATION PROVISION INFORMATION

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE STATUS |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | ABOVE RDP |  | BELOW RDP |  |
| Tlakale Primary | Standpipes | Borehole | Electricity |  | Yes |  | Yes |  | New location Identified for construction of new blocks |
| Mankopane Primary | Standpipes | Borehole | Electricity |  | Yes |  | Yes |  | Provision of water infrastructure |
| Jacob Marwale Primary | Standpipes | Borehole bulk | Electricity | Yes |  | Yes |  |  |  |
| Ngwanamala Secondary | Standpipes | Borehole bulk | Electricity | Yes |  |  | Yes |  | Provision of adequate sanitation infrastructure Provision of water infrastructure |
| Phukubjane Primary | No infrastructure | No source |  |  | Yes | Yes |  |  |  |
| Thobehlale Primary | Standpipes | Borehole bulk | Electricity | Yes |  | Yes |  |  |  |
| Moretlwe Secondary | Standpipes | Borehole bulk | Electricity |  | Yes | Yes |  |  | Provision of water infrastructure |

| CLINIC NAME | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  |  | REMARK & CHALLENGES | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | SIMPLE LATRINE/Below RDP | VIP/RDP | SEPTIC TANK/ WATER BORN |  |  |
| Nchabeleng Clinic | Standpipe, | borehole | Electricity |  | Yes | 4 |  |  |  | Augment the existing scheme |
| Nchabeleng Health Centre | Minor retic | Bulk Borehole | Electricity | Yes |  |  |  |  | Unreliable supply | Augment the existing scheme |

HEALTH FACILITIES EXISTING INFRASTRUCTURE: WATER AND SANITATION

DOMESTIC /HOUSEHOLDS WATER WARD7

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLDS | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  |  |  |  | BOTTLENECKS | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | STORAGE & STAND PIPES |  | NUMBER OF BOREHOLES/ SOURCE |  | SOURCE OF ENERG | ABOVE RDP |  | BELOW RDP |  |  |
| Apel: 1. Ga Maisela | 1260 | No source |  |  |  |  |  |  | Yes |  | Reticulation Yard connection Source: Olifantspoort South |
| 2. Mankotsane |  | 37 standpipes |  |  | 3 x borehole Bulk | Electricity |  | Yes |  |  | Yard connection |
| 3. Thaba Ntsho |  | No source |  |  |  |  |  |  | Yes |  | Reticulation Yard connection Source: Olifantspoort South Long term Solution :Cost recovery |
| 4. Mashung | 203 |  |  |  | Bulk Line yard connection | Electricity |  | Yes |  | Reticulation not covering other sections | Cost recovery Augment the existing yard connections |
| Stydkraal A | 368 | 2 x Borehole reticulation 9 stand pipes Drought relieve |  |  | Diesel | Electricity |  |  | Yes | Capital project failed . Vandalism, Yield Un authorized connections | Feasibility study Upgrade the existing scheme Formalize the unauthorized existing yard connections Connect the existing line to the Olifantspoort bulk line |
| Mapodi/Leshong | 45 | No infrastructure |  |  | None | None |  |  |  |  | Provision of water infrastructure .Yard connection |
| Mooiplaas | 48 | reticulation Drought relieve 7 x stand pipes |  |  | 1 x Borehole | Electricity |  |  | Yes | Inlet network vandalism, Yield. Un authorized connections | Feasibility study Upgrade the existing scheme Formalize the unauthorized existing yard connections Connect the existing line to the Olifantspoort bulk line |
| Stykraal B | 245 | reticulation 20 Stand pipes |  |  | 6 x Boreholes 1 x tank | Electricity |  |  | Yes | Vandalism, Yield Un authorized connections | Feasibility study Upgrade the existing scheme Formalize the unauthorized existing yard connections Connect the existing line to Olifantspoort bulk line |
| Mooiklap | 106 | Reticulation 4x standpipes |  |  |  |  |  |  |  | Vandalism, Yield Un authorized connections | Upgrade the existing scheme Formalize the unauthorized existing yard connections. Connect the existing line to the Olifantspoort bulk line |

DOMESTIC / HOUSEHOLDS SANITATION INFORMATION

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLDS | SANITATION EXISTING INFRASTRUCTURE &STATUS |  | SANITATION INFRASTRUCTURE BACKLOG | NUMBER OF SANITATION FACILITIES NEEDED |
| --- | --- | --- | --- | --- | --- |
|  |  | VIP/RDP | SIMPLE LATRINE BELOW RDP |  |  |
| Apel: |  |  |  |  | Feasibility Study Provide disadvantaged families with ventilated prefabricated latrines |
| 1. Ga Maisela |  |  |  |  |  |
| 2. Thaba Ntsho |  |  |  |  |  |
| 3. Mankotsane | 1260 | 105 | 1055 |  |  |
| 4. Mashung | 203 | 9 | 196 |  |  |
| Stykraal A | 365 | 7 | 357 |  | Feasibility Study Provide disadvantaged families with ventilated prefabricated latrines |
| Mooiplaas | 48 | 23 | 25 |  |  |
| Mapodi Leshong | 45 | 10 | 25 |  |  |
| Stykraal B | 245 | 56 | 184 |  |  |
| Mooiklap | 106 | 2 | 104 |  |  |

SCHOOLS   WATER  AND SANITATION PROVISION INFORMATION

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | STORAGE & STAND PIPES | NUMBER OF BOREHOLES/ SOURCE | SOURCE OF ENERG | ABOVE RDP | BELOW RDP | VIP/ABOVE RDP |  | SIMPLE LATRINE /BELOW RDP |  |
| Moenyane Primary | 2x Standpipes | Borehole | Electricity |  | Yes |  | 12 |  | Provision of water infrastructure |
| Strydkraal Primary | Stand pipes and tanks | Borehole | Electricity | Yes |  | 16 | 16 |  |  |
| Makopole Secondary | 2 x stand pipes | Borehole | Electricity |  | Yes |  | 8 |  | Provision of water infrastructure Provision of adequate sanitation facilities |
| Modipa Agric High | Stand pipes and tanks | Community borehole | Electricity |  | Yes | Yes |  |  | Provision of water infrastructure |
| Nkotsane P.School | 2 stand pipes,2 jojo tanks | 1 x boreholes | Electricity |  | Yes |  | 8 |  | Provision of water infrastructure Provision of adequate sanitation facilities |
| Madithame H . School | none | none |  |  |  |  | 6 |  |  |
| Moloke C. | 3 x stand pipes ,4 x jojo Tanks | Borehole | Electricity | Yes |  | 12 | 12 |  |  |

HEALTH FACILITIES EXISTING INFRASTRUCTURE: WATER AND SANITATION

| CLINIC NAME | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  |  |  |  |  | REMARK & CHALLENGES |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | SIMPLE LATRINE | VIP | SEPTIC TANK | ABOVE RDP | BELOW RDP |  |  |
| Paulus Masha Clinic | 1 x jojo tanks | Borehole | Electricity |  | Yes | Yes | none | Yes | Yes |  |  | Project under construction |
| Mankotsane Clinic | 2 x Jojo tanks | borehole | Electricity |  | Yes |  | 4 |  | Yes |  |  |  |

DOMESTIC /HOUSEHOLDS WATER:WARD 8

| VILLAGES SUBVILLAGES |  | NUMBER OF HOUSEHOLDS | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  |  | BOTTLENECKS | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | STORAGE & STAND PIPES | NUMBER OF BOREHOLES/ SOURCE |  | SOURCE OF ENERG | ABOVE RDP | BELOW RDP |  |  |
| Ga Nkwana: 1.Moshate |  | 2000 | Bulk Line, yard connection 96 x stand pipes | 1 x stand pipes |  | Electricity |  |  | Slope No cost recovery leakage | Implementation of cost recovery. Awareness Campaign O&M –for the sustainability of schemes Reservoir Out flow metering |
| 2. Mashung Avenue |  | 545 | 16 x stand pipes 3x jojo tanks Bulk Line ,yard connections | 2 x borehole |  |  |  |  | Slope No cost recovery Leakage | Implementation of cost recovery. O&M- for the sustainability schemes. |
| 3. Mashung Extension |  |  |  |  |  |  |  |  |  |  |
|  |  |  | No existing line |  |  |  |  |  |  |  |
| 4. Mapodi |  | 350 | Bulk Line 7 x stand pipes |  |  |  |  | Yes | Reticulation not cover the whole area | Extension of reticulation O&M- for the sustainability schemes Source : Olifantspoort South |
| Ga Nchabeleng: 1. Masweneng |  | 10 | Drought relieve:1 x Tanks, 3 stand pipe | 1 x borehole |  | Diesel |  |  | No existing reticulation | O&M- for the sustainability schemes Small reticulation |
| 2. Magotwaneng |  |  | 2x Boreholes Hand pump/Reticulation |  |  |  |  |  | Reticulation awaiting completion of pump station | Extension of reticulation O&M –for the sustainability of schemes |
| Magabaneng |  |  | Reticulation |  | Bulk |  |  |  | Reticulation awaiting completion of pump station | O&M –for the sustainability of schemes |
| Mohlaletse: Matsemela |  | 297 | Reticulation ,RDP standard |  | Bulk 1 x borehole | Diesel Bulk |  |  | Reticulation awaiting completion of pump station | O&M –for the sustainability of schemes |
| Maisela Maime |  | 104 | Reticulation ,RDP standard |  | Bulk |  |  |  |  |  |
| Maisela: Mahlabaphooko |  | 333 | RDP Standard (bulk line) 32 x Stand pipes |  | Bulk 3 borehole |  |  |  | Leakages | O&M –for the sustainability of schemes. Awareness campaigns Long term solution :Yard connection –feasibility study required |
| Summary | 64 old street stand pipes not in use |  |  |  |  |  |  |  |  |  |

DOMESTIC/HOUSEHOLDS SANITATION INFORMATION

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLDS | SANITATION EXISTING INFRASTRUCTURE |  | SANITATION EXISTING INFRASTRUCTURE & STATUS | STRATEGY |
| --- | --- | --- | --- | --- | --- |
|  |  | VIP | SIMPLE LATRINE |  |  |
| GA NKWANA: 1. Moshate | 2000 210 | 125 | 300 |  | Feasibility Study Provide disadvantaged families with ventilated prefabricated latrines |
| 2. Mashung | 545 | 53 | 493 |  |  |
| 3. Mapodi | 173 |  |  |  |  |
| GA NCHABELENG : 1. Masweneng | 10 |  | 10 | 10 | Feasibility Study Provide disadvantaged families with ventilated prefabricated latrines |
| 2. Magotwaneng |  |  |  |  |  |
| Magabaneng |  |  |  |  |  |
| MOHLALETSE: 1.Matsemela | 297 |  | 297 | 297 | Feasibility Study Provide disadvantaged families with ventilated prefabricated latrines |
| 2. Maisela Maime | 104 |  | 14 | 81 |  |
| Maisela Mahlabaphoko | 333 | 108 | 221 |  | Feasibility Study Provide disadvantaged families with ventilated prefabricated latrines |

SCHOOLS   WATER AND SANITATION PROVISION INFORMATION

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | STORAGE &STAND PIPES | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | VIP/ABOVE RDP | SIMPLE LATRINE BELOW RDP | Augment existing facilities to meet RDP standard |
| Nkwana Primary | 1 x jojo tank ,1 x stand pipe | Bulk | Electricity | Yes |  | 8 | 4 | Augment existing facilities to meet RDP standard |
| Maisela Primary | 1 x jojo tank | Borehole | Electricity | Yes |  | 8 |  | Augment existing facilities to meet RDP standard |
| Mafoufale Secondary |  |  |  | Yes |  | 8 |  | Augment existing facilities to meet RDP standard |
| Frank Mashile | Yard connection 2 x jojo tanks | Bulk Borehole | Electricity | Yes |  | 8 |  | Augment existing facilities to meet RDP standard |
| Mmaphotle Primary | Yard connection | Bulk Borehole | Electricity | Yes |  |  | 5 | Augment existing facilities to meet RDP standard |
| Moroka Lebole Secondary | Yard connection , 2 x jojo tanks | Borehole | Electricity | Yes |  |  | 4 | Additional sanitation facilities required to meet RDP standard |
| Naka Mmakgomo Secondary | 1 x stand pipe. Yard Connection | Bulk | Electricity | Yes |  | 8 |  | Additional sanitation facilities required to meet RDP standard |
| Kgwedi Primary | Stand pipes 1x jojo tank | Borehole | Electricity |  | Yes |  | 02 | Additional sanitation facilities required to meet RDP standard |
| Tshweele Primary | 1 x jojo tank | Borehole | Electricity |  | Yes |  | 2 | Additional sanitation facilities required to meet RDP standard |

HEALTH FACILITIES EXISTING INFRASTRUCTURE: WATER AND SANITATION

| Clinic Name | WATER EXISTING NFRASTRUCTURE & STATUS |  |  |  | SANITATION EXISTING NFRASTRUCTURE & STATUS |  |  | Sanitation |  | Remark & Challenges | Strategy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Water | Energy | Above RDP | Below RDP | Simple Latrine | VIP | Septic Tank | Above RDP | Below RDP |  |  |
| Nkwana Clinic | 2x jojo tanks, 2 x Stand pipes | Electricity | Yes |  | 4 | 8 | 1 | Yes |  |  |  |

DOMESTIC/ HOUSHOLDS WATER: WARD 9

| VILLAGES SUBVILLAGES | NUMBER OF HOUSEHOLDS | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  |  | BOTTLENECKS | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | STORAGE &STAND PIPES | SOURCE |  | ENERGY | ABOVE RDP | BELOW RDP |  |  |
| Pelangwe | 201 | Bulk line at RDP Standard 10 Stand pipes |  |  | Electricity |  |  |  | Feasibility study Yard connections Awareness campaign |
| Indie | 294 | 19 x stand pipe | Bulk line at RDP Standard |  | Electricity | Yes |  | Slope Pressure problem Scheme not covering the whole village | Feasibility study Yard connections Awareness campaign Extension of line Utilization of the existing reservoir to maintain enough pressure |
| Malogeng | 234 | Bulk line at RDP Standard 1 x reservoir 35 x Stand pipe |  |  | Electricity Bulk |  |  |  | Long term solution s: Feasibility study Yard connections Awareness campaign Cost recovery Short term solution: Additional stand pipes Extension of line |
| Maruping | 189 | 12 stand pipes | 1x Borehole reticulation at RDP standard |  | Diesel |  | Yes | Borehole supply not sufficient | Short term solution :Augment the borehole by: Drill a new borehole Rehabilitate the borehole Long term :Construction of Bulk line from Olifantspoort |
| Mphaaneng | 88 | 26 x stand pipe |  | Bulk | Electricity | Yes |  |  | Short term solution : Additional stand pipes Extension of line Long term solution: Yard connection: Cost recovery |
| Modimolle | 92 | Bulk line at 24 stand pipes |  | Bulk | Electricity | Yes |  |  | Short term solution : Additional stand pipes Extension of line Long term solution: Yard connection: Cost recovery |
| Mogabane | 259 | Reticulation 13 stand pipes |  | Borehole | Diesel Electricity | Yes |  |  | Extension of reticulation |
| Malomanye | 481 | Reticulation 23 x Stand pipes |  | Borehole Bulk | Electricity |  | Yes |  | Augment existing infrastructure |
| Mafeana | 391 | Bulk line RDP standard Drought relief |  | Bulk | Electricity Petrol. |  | Yes | Unreliable source | Connect of the drought relief inlet to the bulk line to address source issue. |
| Mashilabele | 8 | Hand pump |  |  |  |  | Yes | Hand pump not functional | Drought relief project Long term : Bulk supply. |
| Petja | 8 | 1 x stand pipe |  | Bulk | Electricity |  | Yes |  | Augment up to RDP standard . |

DOMESTIC /HOUSEHOLDS SANITATION INFORMATION

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLDS | NUMBER OF SANITATION FACILITIES PROVIDED |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- |
|  |  | VIP/RDP | SIMPLE LATRINE/BELOW RDP | SANITATION INFRASTRUCTURE BACKLOG |  |
| Pelangwe | 201 | 54 | 147 |  | Feasibility studies –Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| Indie | 294 | 54 | 234 |  |  |
| Malogeng | 234 | 35 | 134 |  |  |
| Maruping | 189 | 28 | 160 | 103 |  |
| Mphaaneng | 88 | 7 | 75 |  |  |
| Modimolle | 92 | 9 | 82 |  |  |
| Mogabane | 251 | 21 | 196 | 88 |  |
| Malomanye | 481 | 43 | 368 |  |  |
| Mafeana | 391 | 11 | 4 |  |  |
| Mashilabela | 8 |  | 8 |  |  |
| Petja | 8 |  | 8 |  |  |

SCHOOLS   WATER  AND SANITATION PROVISION INFORMATION

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | STORAGE & STAND PIPES | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | VIP/E RDP | SIMPLI LATRINE/BELOW RDP |  |
| Selebalo Secondary | 1x Stand pipes | Bulk | Electricity | Fair |  |  | 5 | Augment up the existing infrastructure up to RDP standard |
| Mphaaneng Primary | 1 x Stand pipes | Bulk | Electricity | Fair |  |  | 6 | Augment up the existing infrastructure up to RDP standard |
| Mafene H.Primary | 2 x Stand pipes | Bulk | Electricity | Fair |  | 4 | 2 | Augment up the existing infrastructure up to RDP standard |
| Monare Sec | No infrastructure |  |  |  | Yes | 4 | 2 | Provision of water infrastructure –up to the required standard |
| Phuti Tlou High School | Stand pipe | Bulk | Electricity |  | Yes |  | 10 | Augment up the existing infrastructure up to RDP standard |
| India Primary | Stand pipes | Bulk | Electricity |  | Yes |  | 4 |  |
| Mohlotlwane Secondary | Stand pipe | Bulk Hand pump |  |  | Yes |  | 10 | Provision of adequate sanitation facilities for learners. Upgrade the existing infrastructure |
| Pelangwe Primary | Stand pipe | Bulk | Electricity |  | Yes |  | 4 | Provision of adequate sanitation facilities. Upgrade the existing infrastructure |
| Phoko H.Primary | 1 x Stand pipe | borehole | Electricity |  | Yes |  | 4 | Augment up the existing infrastructure up to RDP standard |
| Mahudu H.Primary | 1 x Stand pipe | Main line | Electricity |  | Yes |  | 4 | Augment up the existing infrastructure up to RDP standard |
| Modimolle Primary | Stand pipe | Bulk | Electricity |  | Yes |  | 2 | Provision of adequate sanitation facilities . Upgrade the existing infrastructure |

DOMESTIC/ HOUSHOLDS WATER:WARD 10

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLDS | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | BOTTLENECKS | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | STORAGE & STAND PIPES | SOURCE | ENERGY | ABOVE RDP | BELOW RDP |  |  |
| Ga-Selepe Main Scheme | 826 | 22 stand pipes 2 x storage reticulation, | Borehole | Electricity |  |  | Leakages. Borehole supply not reliable . Not RDP standard. | O& M-to sustain the scheme Augmentation of the whole scheme |
| Fanang Diatla |  | reticulation | Borehole | Electricity |  |  | Leakages . Borehole supply not reliable. | O& M-to sustain the scheme Augmentation of the whole scheme |
| Sekhutlong |  | Reticulation 1 stand pipe | Borehole | Diesel |  |  | Leakages. Borehole supply not reliable. | Augmentation of the whole scheme O& M-to sustain the scheme Electrification of the borehole |
| Madingwane/Mogoleng |  | Drought relief 2 x stand pipe |  | Diesel |  |  | Below RDP. | Upgrading of the whole scheme-up to RDP standard O& M-to sustain the scheme Electrification of the borehole |
| Manotwane | 311 | Reticulation, 5 stand pipes | Borehole | Diesel | Yes |  | Below RDP | O& M-to sustain the scheme Electrification of the borehole. Upgrading of the whole scheme-up to RDP standard. |

DOMESTIC/HOUSEHOLDS SANITATION INFORMATION

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLD | SANITATION EXISTING INFRASTRUCTURE & STATUS |  | SANITATION INFRASTRUCTURE BACGLOG | STRATEGY |
| --- | --- | --- | --- | --- | --- |
|  |  | VIP/RDP | SIMPLE LATRINE /BELOW RDP |  |  |
| GaSelepe : Main Scheme | 826 | 134 | 630 | 62 | Provide disadvantaged families with ventilated prefabricated latrines |
| Fanang Diatla |  |  |  |  |  |
| Sekhutlong |  |  |  |  |  |
| Madingwane |  |  |  |  |  |
| Manotwane | 311 | 51 | 360 |  |  |

SCHOOLS   WATER  AND SANITATION PROVISION INFORMATION

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION STATUS |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | VIP/E RDP |  | Simple Latrine /Below RDP |  |
| Selepe H. Primary | 3 stand pipes ,3 x tanks | Borehole | Petrol | Yes |  |  |  |  | Upgrade the existing infrastructure |
| Lefakgomo Sec | No infrastructure | No source |  |  | Yes |  | Yes |  | Provision of adequate sanitation facilities for learners. Upgrade the existing infrastructure |
| Mamokgalake H. Primary | No infrastructure | No source |  |  | Yes |  | Yes |  | Provision of adequate sanitation facilities for learners. Upgrade the existing infrastructure |
| Manotwane Primary | Stand pipe | Borehole | Diesel |  | Yes |  | Yes |  | Provision of adequate sanitation facilities for learners. Upgrade the existing infrastructure |
| Tlou Phuti Secondary |  | Borehole | Diesel |  | Yes |  | Yes |  | Provision of adequate sanitation facilities for learners . Upgrade the existing infrastructure |
| Sejadipudi Primary |  |  |  |  | Yes |  | Yes |  |  |

| CLINIC NAME | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  |  |  |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | SIMPLE LATRINE | VIP | SEPTIC TANK | ABOVE RDP | BELOW RDP |  |  |
| Manotwane Clinic | Stand pipe | Borehole | Petrol |  | Yes | 2 |  |  |  | Yes | Upgrade existing infrastructure up to RDP level |  |
| Selepe Clinic |  | borehole | electricity |  | Yes |  | Yes | Yes |  | Yes |  |  |

CLINICS WATER AND SANITATION PROVISION INFORMATION

DOMESTIC/ HOUSHOLDS WATER:WARD 11

| VILLAGES SUBVILLAGES | NUMBER OF HOUSEHOLDS | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  |  |  | BOTTLENECKS BELOW RDP | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | WATER | SOURCE |  | ENERGY |  | ABOVE RDP | BELOW RDP |  |  |
| Mosotse | 57 | Reticulation 3 x standpipes 1 x tank Pump house | Borehole |  | Diesel |  |  |  | Leakages | - Electrification of boreholes - O&M |
| Phashaskraal :Main scheme |  | Borehole reticulation at RDP Standard |  |  | Diesel |  |  |  |  | - Electrification of boreholes - O&M |
| Sekhutlong /Ka-Gathimoty |  | Drought relief ,One tank and two stand pipe |  |  | Diesel |  |  |  | No reticulation | - Electrification of boreholes - O&M - Upgrade existing structure up to RDP standard |
| Mashegeng |  | Hand pump |  |  |  |  |  |  | No reticulation | Drought relief :Drilling and Equipping of borehole and four draw offs |
|  |  |  |  |  |  |  |  |  | No reticulation |  |
| Phasha Selatole: Main scheme Sebitsa | 531 | Borehole reticulation 4 stand pipe Drought relief scheme 3 x tanks 5 boreholes |  |  | Diesel |  |  |  | No energy source | Energising/Electrification of borehole Supply of Generator |
| Malaeneng |  | Borehole Reticulation |  |  | Electricity |  |  |  | No energy source | Energising/Electrification of borehole |
| Seokodibeng |  | Reticulation at RDP standard | 2xBorehole | Electricity |  |  |  |  | Vandalism | Re –installation of transformer Awareness Campaigns |
| GaMampa | 214 | Reticulation at RDP standard 8 x stand pipes | Borehole |  |  |  |  |  |  | - Electrification of boreholes - O&M - Upgrade existing structure up to RDP standard |
| Ledingwe: Main scheme | 221 | Reticulation at RDP standard 12 x stand pipes | Borehole | Diesel |  |  |  |  | Capital Project failed | Completion of the long stalled project |
| Ramallane |  | Drought relief 2 x Stand pipes |  | Diesel |  |  |  |  |  | Upgrade of the existing infrastructure up to RDP standard |
| Senthlane |  | Drought relief 2 x stand pipes |  | Diesel |  |  |  |  |  | Upgrade of the existing infrastructure up to RDP standard |

DOMESTIC /HOUSEHOLDS SANITATION INFORMATION

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLD | SANITATION EXISTING INFRASTRUCTURE & STATUS |  | SANITATION INFRASTRUCTURE BACKLOG | STRATEGY |
| --- | --- | --- | --- | --- | --- |
|  |  | VIP/RDP | SIMPLE LATRINE/BELOW RDP |  |  |
| Mosotse | 57 |  | 49 | 8 | Feasibility study –Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| Phashaskraal: Main scheme | 204 | 14 | 47 | 147 |  |
| Sekhutlong |  |  |  |  |  |
| Mashegeng |  |  |  |  |  |
| PhashaSelatole:Main scheme Sebitsa | 531 |  |  |  |  |
| Malaeneng |  |  |  |  |  |
| Seokodibeng | 433 | 49 | 69 |  |  |
| GaMampa | 214 |  |  | 214 |  |
| Ledingwe:Main scheme | 221 | 59 | 70 | 103 |  |
| Ramallane |  |  |  |  |  |
| Senthlane |  |  |  |  |  |

SCHOOLS   WATER AND SANITATION PROVISION INFORMATION

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER |  | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | ABOVE RDP |  | BELOW RDP |  |
| Tswereng Primary | Stand pipes |  | Community source | Diesel |  | Yes |  | Yes |  | Augment the existing infrastructure by ensuring adequate storage. Provision of adequate sanitation facilities. |
| Modiadie High | No infrastructure |  |  |  |  | Yes |  | Yes |  |  |
| Seroletshidi Sec | No infrastructure |  |  |  |  | Yes |  | Yes |  |  |
| Malegase Prim | Stand pipe |  | Borehole | Diesel |  | Yes |  | Yes |  |  |
| Motjatjana Primary |  |  | Community borehole | Diesel |  | Yes |  | Yes |  |  |
| Makgalanoto Primary | No infrastructure |  | No source |  |  | Yes | Yes |  |  | Provision of water by drilling ,equipping and installation of stand pipes Provision of adequate sanitation facilities |
| Selatole High | Stand pipe |  | Borehole | Petrol |  | Yes |  | Yes |  | Provision of water by drilling ,equipping and installation of stand pipes Provision of adequate sanitation facilities |
| Poo Sec | Stand pipe |  | Borehole | Diesel |  | Yes |  | Yes |  | Provision of water by drilling ,equipping and installation of stand pipes Provision of adequate sanitation facilities |
| Manku Primary | stand pipes |  | Borehole | Diesel |  | Yes | Yes |  |  |  |
| Kgagudi Primary | Stand pipe |  | Community borehole | Diesel |  | Yes | Yes |  |  |  |
| Ramoko primary |  | Borehole |  | Diesel | Yes |  |  | Yes |  |  |

CLINICS WATER AND SANITATION PROVISION INFORMATION

| CLINIC NAME | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION EXISTING INRASTRUCTURE & STATUS |  |  |  |  |  | REMARK & CHALLENGES | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | SIMPLE LATRINE | VIP | SEPTIC TANK | ABOVE RDP | BELOW RDP |  |  |  |
| Phasha Clinic |  | Borehole | Electricity |  |  |  |  |  |  |  |  | Challenge :Only 2 VIP toilets exit | Additional sanitation facilities required to meet the required standard |

DOMESTIC/ HOUSHOLDS WATER: WARD 12

| VILLAGES SUBVILLAGES | NUMBER OF HOUSEHOLDS | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | BOTTLENECKS | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP |  |  |
| Mabulela 1. Mabulela A | 163 | Drought relief |  | Diesel |  |  |  | - Electrification of the borehole |
| 2. Mabulela B/ Matshakaneng |  | Reticulation at RDP Standard 4 x Reservoirs 114 x stand pipes | 4 x Borehole | Diesel |  |  | Defects experienced within retention period | - Electrification of borehole |
| 3. Mohlahlaneng | 352 | 26 x stand pipes Reticulation at RDP standard | 4 x Borehole | Electricity |  |  | Vandalism | Awareness campaign |
| Bogalatladi | 237 | Reticulation at RDP Standard RESERVIORS 74 x Stand pipes | 5 x Borehole 1 Pump house | Diesel Electricity |  |  | Leakages and pressure | - Rectification of defects - Electrification of borehole |
| Mashikwe | 51 | Drought relief and reticulation 2x stand pipes 2 x tanks | 3 x Borehole | Diesel Electricity |  |  | Vandalism Below RDP standard | - Electrification of borehole - Awareness campaign |
| Sefateng Mphotheleng | 185 | reticulation at RDP Standard 1x Reservoir | 2x Borehole | Electricity |  |  | Quality of water | Installation of package plant to neutralized the soupiness of water (test to be done to identify the proper equipments) Connect scheme to Olifantspoort from Malomanye |

HOUSEHOLDS SANITATION INFORMATION

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLD | NUNBER OF SANITATION FACILITY PROVIDED |  | SANITATION BACKLOG INFRASTRUCTURE | STRATEGY |
| --- | --- | --- | --- | --- | --- |
|  |  | VIP/RDP | SIMPLE LATRIN/ BELOW RDP |  |  |
| Mabulela:Mabulela A | 270 | 38 | 232 |  | Feasibility study –Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| Mabulela B/ Matshakaneng |  |  |  |  |  |
| Bogalatladi | 237 | 19 | 284 | 56 |  |
| Mashikwe | 51 | 9 | 38 |  |  |
| Sefateng Mphotheleng | 185 | 14 | 155 | 16 |  |
| Mohlahlaneng | 352 |  | 222 |  |  |

SCHOOLS   WATER AND SANITATION PROVISION INFORMATION

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION INFRASTRUCTURE & STATUS |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | VIP/ RDP | SIMPLE LATRINE /BELOW RDP |  |
| Motsepe Primary | Stand pipes ,tanks | Borehole | Electrical | Yes |  | Yes |  | O&M |
| Potlake Secondary |  | Borehole | Electrical | Yes |  | Yes |  | O&M |
| Bogalatladi Primary |  | Borehole | Electrical | Yes |  | Yes |  | O&M |
| Serokolo Secondary |  | Borehole | Electrical | Yes |  | Yes |  | O&M |
| Mafise Primary |  | Borehole | Electrical | Yes |  | Yes |  | O&M |

CLINICS WATER AND SANITATION PROVISION INFORMATION

| CLINIC NAME | WATER EXISTING INFRASTRUCTURE &STATUS |  |  |  |  | SANITATION EXISTNG INFRASTRUCTURE & STATUS |  |  | SANITATION |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | WATER | SOURCE | ENERGY | ABOVE RDP | BELOW RDP | SIMPLE LATRINE | VIP | SEPTIC TANK |  | ABOVE RDP | BELOW RDP |  |
| Motsepe |  | Borehole | Electrical |  |  | Yes |  | Yes | Yes |  |  |  |

DOMESTIC/ HOUSHOLDS WATER: WARD 13

| VILLAGES/SUBVILLAGES | NUMBER OF HOUSEHOLDS | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  |  | BOTTLENECKS | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | STORAGE & STANDPIPES | SOURCE | ENERGY |  | ABOVE RDP | BELOW RDP |  |  |
| Shubushubung | 40 | Drought relief ,four tanks and two stand pipes |  | Diesel |  |  |  |  | Augment scheme O&M-to sustain the project |
| Rostock | 149 | 1 x Reticulation 29 x stand pipes 2x storage | 2 x Borehole | Diesel |  | Yes |  | Vandalism | Electrification of borehole O& M Awareness Campaigns |
| Mahlabeng Mahlabeng New Stand | 104 | Reticulation 38 x stand pipes 1x reservoir | 1 x Borehole | Diesel |  | Yes |  | Unauthorized yard connections | Electrification of borehole O& M Awareness Campaigns Formalization of existing yard connections |
|  | 74 | Reticulation 24 stand pipes | Borehole |  |  |  |  |  |  |
| Mooilek | 156 | Reticulation 7 x tanks 21 stand pipes. Pump house. | 3 x Borehole | Electricity |  | Yes |  | Unauthorized yard connection | O& M Awareness Campaigns Formalization of existing yard connection |
| Tjibeng | 351 | 4 Borehole reticulation at RDP standard 30 Stand pipe | Drought relief | Diesel Petrol |  |  |  | Inclined Slope | Electrification of boreholes Booster pump O&M |
| Monametse | 212 | Reticulation. Drought relief. 4x stand pipe. | 2 x Borehole | Electricity from the nearest school Diesel |  |  |  | Leaking storage Source not reliable/no underground water | O& M Electrification of borehole Exploration of availability of water from other villages /sources |
| Mokgotho |  | Drought relief ,two tanks and two stand pipes . | 1x borehole | Electricity |  |  |  | Below RDP standard | Upgrade of an existing infrastructure up to RDP Standard |

HOUSEHOLDS SANITATION INFORMATION

| VILLAGES:SUBVILLAGES | NUMBER OF HOUSEHOLD | SANITATION EXISTING INFRASTRUCTURE & STATUS |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- |
|  |  | SIMPLE LATRINE/BELOW RDP | VIP/RDP | NUMBER OF HOUSE HOLDS WITHOUT |  |
| Shubushubung | 40 | 29 |  | 11 | Feasibility study –Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| Rostock | 149 | 74 | 24 | 51 | Feasibility study –Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| Mahlabeng | 104 | 85 | 17 | 2 | Feasibility study –Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| Mahlabeng New Stand | 74 | 24 |  | 50 |  |
| Mooilek | 156 | 53 | 31 | 72 | Feasibility study –Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| Tjibeng | 351 | 101 | 157 | 93 | Feasibility study –Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| Monametse | 212 | 112 | 13 | 87 | Feasibility study –Geotech Provide disadvantaged families with ventilated prefabricated latrines |
| Mokgotho |  |  |  |  | Feasibility study –Geotech Provide disadvantaged families with ventilated prefabricated latrines |

SCHOOLS   WATER AND SANITATION PROVISION INFORMATION

| NAME OF SCHOOL | WATER EXISTING INFRASTRUCTURE & STATUS |  |  |  |  | SANITATION EXISTING INFRASTRUCTURE & STATUS |  |  | STRATEGY |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | WATER OF SOURCE | ENERGY | ABOVE RDP | BELOW RDP | VIP/ RDP |  | SIMPLE LATRI NE/BELOW RDP |  |
| Monametse Primary | 2 x Jojo 2 x stand pipe | Borehole |  |  |  |  | Below |  |  |
| Mogale Primary | Stand pipe | Community source | Electricity |  | Yes |  | Yes |  | Augment existing sanitation facilities Augment of the existing water infrastructure |
| Lebelo Primary | No infrastructure | No source |  |  | Yes |  | Yes |  | Augment existing sanitation facilities Augment of the existing water infrastructure |
| Nyaku Sec | Hand pump | Borehole | Electricity | Yes |  | Yes |  |  | Augment of the existing water infrastructure |
| Morwaswi | 1 x stand pipe | Community borehole |  |  | Yes |  | Yes |  | Augment existing sanitation facilities Augment of the existing water infrastructure |
| Matianyane | Stand pipe | Community Borehole | Diesel | Fair |  |  | Fair |  | Augment existing sanitation facilities Augment of the existing water infrastructure |
| Malengine Sec. | Stand pipe ,1 x jojo tank | Borehole | Electricity |  | Yes |  | Yes |  | Augment existing sanitation facilities Augment of the existing water infrastructure |
| Kwano Primary | Stand pipe | Community Borehole | Diesel | Fair |  |  |  |  | Provision of Sanitation facilities |